XD-S1500Z Technical Specifications

Wuhan X-Dynamics Co., Ltd.

February 2025

Revision History

| Version Number | Revision Date | Revised Chapter | Revision Notes |
| --- | --- | --- | --- |
| V1.1.1 | 2025/02/27 | Entire Chapter | First edition |
| V1.1.2 | 2025/03/4 | Chapter 1 |  |
| V1.1.3 | 2025/09/1 | Chapter 1.3 | Update some parameters and structural drawings |
| V1.1.4 | 2025/09/22 | Chapter 5 | Modify baud rate command |

Copyright Notice

Copyright © Wuhan X-Dynamics Co., Ltd. All rights reserved.

Confidentiality Statement

The information contained in this document (including any attachments) is confidential. The recipient understands that this document is confidential and shall not be used for any purpose other than the specified purpose, nor shall it be disclosed to any third party.

Contents

1. Product Overview	4

1.1. Application Areas	4

2. Specifications and Parameters	4

2.1. Product Physical Diagram	4

2.2. Product Specifications	5

3. Structural Description and Interface Definition	6

3.1. Structural Drawings	6

3.2. Hardware Interface Definition	7

3.3. Basic Performance Measurement	7

4. Electrical Characteristics	8

4.1. Power Supply and Power Consumption	8

4.2. Reliability Standards	8

5. Control Commands	9

5.1. UART Interface	9

5.2. Control Commands	9

5.2.1. Single Measurement Command	9

5.2.2. Continuous Measurement Command	10

5.2.3. Continuous Measurement Stop Command	10

5.2.4. Baud Rate Setting (Immediate Effect)	11

5.2.5. Device Information Retrieval	11

6. Appendix	11

6.1. System Integration Guide	11

6.2. Usage Recommendations	12

XD-S1500Z Technical Specifications

Product Overview

The XD-S1500Z laser rangefinder module is developed based on a 905nm semiconductor laser, featuring a compact size, lightweight design, and long measurement range. Its applications include handheld rangefinders, micro drones, and rangefinder sights. It is equipped with a UART (TTL protocol) data transmission interface, providing upper-level software and communication protocol command sets to facilitate user-defined development.

Product Features of XD-S1500Z:

• Custom-developed high-precision calibration and calibration algorithms;

• Compact size and lightweight design;

• Long measurement range;

## Application Areas

XD-S1500Z has a wide range of applications:

• Handheld rangefinders, micro drones, rangefinder sights, etc.

# Specifications and Parameters

## Product Physical Diagram

The actual product image is shown below:

Product Specifications

The product specifications are as follows:

| Name | Performance Indicators | Indicator Parameters | Remarks |
| --- | --- | --- | --- |
| key indicators | Range | 10~1500m |  |
|  | Accuracy | ±1m (d≤400 meter)； ±(d*0.4%)m (d＞400 meter） |  |
|  | Distance measurement frequency | ≤2~4Hz |  |
|  | Laser emission angle | 7*3mrad |  |
|  | Emission aperture | 10×7.5mm |  |
|  | Reception aperture | 15×10mm |  |
|  | Wavelength | 905nm |  |
|  | Safety level | Class I Eye safety |  |
| system parameters | Working environment | Indoor & Outdoor |  |
|  | Working temperature | -30℃ ~ 60℃ |  |
|  | Storage temperature | -30℃ ~ 80℃ |  |
|  | Impact level | 1200G.1ms |  |
|  | Structural dimensions | ≤24.6mm × 13.55mm × 25.72mm |  |
|  | Weight | 10±0.5g |  |
| electrical | Communication interface | UART(TTL-3.3V) |  |
|  | Power interface | SM06B-SURS-TF(LF)(SN) |  |
|  | Supply voltage | DC3V~5V |  |
|  | Average power consumption | ≤600mW |  |
|  | Baud rate | 115200bps(default) |  |

Structural Description and Interface Definition

Structural Drawings

The structural dimensions of the XD-S1500Z product are shown below:

## Hardware Interface Definition

The product hardware interface structure diagram is shown below:

The interface specifications corresponding to the product interface diagram are shown in the table below:

- Electrical interface: UART (TTL-3.3V)

- Connector model: 6PIN 0.8mm pitch terminal

Connector wire sequence table

| Pin | Definition | Description |
| --- | --- | --- |
| 1 | GND | Serial port ground/power ground |
| 2 | VCC | Power supply: 3V to 5V, maximum cannot exceed 5.5V |
| 3 | NC | NC |
| 4 | UART_TX | Serial port transmitter, TTL 3.3V |
| 5 | UART_RX | Serial port receiver, TTL 3.3V |
| 6 | POWER_ON | Power switch (High level enables power, low level turns off power, where high level is power level) |

## Basic Performance Measurement

The measurement range of this product is defined under the following conditions:

(1) The measurement target has moderate reflectivity: such as building walls;

(2) The measurement target's reflective surface is perpendicular to the laser emission direction;

(3) The weather conditions are clear but not under direct sunlight;

Electrical Characteristics

Power Supply and Power Consumption

The power supply and power consumption parameters of the XD-S1500Z product are shown in the table below:

Power Supply Parameters and Specifications Table

| Supply voltage | DC3V~5V |
| --- | --- |
| Average power consumption | ≤600mW |
| Peak power consumption | ≤1.65W |
| Standby power consumption | ≤1.5 mW (EN disabled) |

Reliability Standards

The reliability standard parameters for the XD-S1500Z product are shown in the table below:

| Operating temperature | -30℃~60℃ |
| --- | --- |
| Storage temperature | -30℃~80℃ |
| Impact level | 1200G.1ms |

Control Commands

UART Interface

• Baud rate: 115200 bps (default)

• Start bit: 1 bit

• Data bits: 8 bits

• Stop bit: 1 bit

• Parity bit: None

• Flow control: None

Control Commands

Transmit checksum = byte 3 + byte 4 + byte 5 + byte 6 + byte 7;

Receive checksum = byte 1 + byte 2 + byte 3 + byte 4 + byte 5 + byte 6 + byte 7;

Single Measurement Command

- The single measurement command is as follows；

Single measurement command

| Byte 1 | Byte 2 | Byte 3 | Byte 4 | Byte 5 | Byte 6 | Byte 7 | Byte 8 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 0x55 | 0xAA | 0x88 | 0xFF | 0xFF | 0xFF | 0xFF | checksum |

Single measurement return value

| Byte 1 | Byte 2 | Byte 3 | Byte 4 | Byte 5 | Byte 6 | Byte 7 | Byte 8 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 0x55 | 0xAA | 0x88 | Status | 0xFF | DATA_H | DATA_L | checksum |

Status = 0，Single measurement failure； DATA_H=0xFF，DATA_L=0xFF； Status = 1，Single measurement success；DATA_H=High byte of measurement result；DATA_L=Measurement result low byte；

Continuous Measurement Command

- The continuous measurement command is as follows:；

Continuous measurement command

| Byte 1 | Byte 2 | Byte 3 | Byte 4 | Byte 5 | Byte 6 | Byte 7 | Byte 8 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 0x55 | 0xAA | 0x89 | 0xFF | 0xFF | 0xFF | 0xFF | checksum |

Continuous Measurement Return Values

| Byte 1 | Byte 2 | Byte 3 | Byte 4 | Byte 5 | Byte 6 | Byte 7 | Byte 8 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 0x55 | 0xAA | 0x89 | Status | 0xFF | DATA_H | DATA_L | checksum |

Status = 0，Multiple measurement failures； DATA_H=0xFF，DATA_L=0xFF； Status = 1，Multiple measurements successful；DATA_H=High byte of measurement result；DATA_L=Measurement result low byte；

Continuous Measurement Stop Command

- The continuous measurement stop command is as follows；

Continuous measurement stop command

| Byte 1 | Byte 2 | Byte 3 | Byte 4 | Byte 5 | Byte 6 | Byte 7 | Byte 8 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 0x55 | 0xAA | 0x8E | 0xFF | 0xFF | 0xFF | 0xFF | checksum |

Return value when continuous measurement stops

| Byte 1 | Byte 2 | Byte 3 | Byte 4 | Byte 5 | Byte 6 | Byte 7 | Byte 8 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 0x55 | 0xAA | 0x8E | Status | 0xFF | 0xFF | 0xFF | checksum |

Status = 0，Close multiple measurement failures； Status = 1，Close multiple measurements successful；

Baud Rate Setting (Immediate Effect)

Baud rate transmission settings

| Byte 1 | Byte 2 | Byte 3 | Byte 4 | Byte 5 | Byte 6 | Byte 7 | Byte 8 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 0x55 | 0xAA | 0x01 | 0xFF | 0xFF | 0xFF | 0x01 | checksum |

Byte  7，Byte  8：

0x01,0xFF——Baud Rate 9600bps；0x02,0x00——Baud Rate 14400bps；

0x03,0x01——Baud Rate 19200bps；0x04,0x02——Baud Rate 38400bps；

0x05,0x03——Baud Rate 57600bps；0x06,0x04——Baud Rate 115200bps；

0x07,0x05——Baud Rate 128000bps；0x08,0x06——Baud Rate 230400bps；

Baud rate setting return

| Byte 1 | Byte 2 | Byte 3 | Byte 4 | Byte 5 | Byte 6 | Byte 7 | Byte 8 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 0x55 | 0xAA | 0x01 | Status | 0xFF | 0xFF | 0x01 | checksum |

Status=0x00，Baud rate setting failed； Status=0x01，Baud rate setting successful；

0x01——Baud rate 9600bps；0x02——Baud rate 14400bps；0x03——Baud rate 19200bps；

0x04——Baud rate 38400bps；0x05——Baud rate 57600bps；0x06——Baud rate 115200bps；

0x07——Baud rate 128000bps；0x08——Baud rate 230400bps；

Device Information Retrieval

Device Information Retrieval

| Byte 1 | Byte 2 | Byte 3 | Byte 4 | Byte 5 | Byte 6 | Byte 7 | Byte 8 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 0x55 | 0xAA | 0xA0 | 0xFF | 0xFF | 0xFF | 0xFF | checksum |

Device information retrieval return

| Byte 1 | Byte 2 | Byte 3 | Byte 4 | Byte 5 | Byte 6 | Byte 7 | Byte 8 | Byte 8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0x55 | 0xAA | 0xA0 | Status | 0xFF | 0x58 | 0x44 | 0x02 | checksum |

Status=0x00，Device information retrieval return； Status=0x01，Device information retrieved successfully；

Power-on startup completion information

Power-on startup completion information

| Byte 1 | Byte 2 | Byte 3 | Byte 4 | Byte 5 | Byte 6 | Byte 7 | Byte 8 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 0x55 | 0xAA | 0xA1 | 0xFF | 0xFF | 0xFF | 0xFF | checksum |

Appendix

System Integration Guide

Before selecting the XD-S1500Z camera for development, please contact Wuhan X-Dynamics Co., Ltd. to obtain technical specifications and other relevant information. Through preliminary evaluation, debugging, and verification, you can determine whether it meets mass production requirements.

Recommended Process:

• Read the XD-S1500Z Technical Specifications;

• Purchase and obtain an XD-S1500Z sample unit through official channels;

• Contact personnel from Wuhan Jidong Intelligent Technology Co., Ltd. prior to development to obtain relevant materials;

• Select an appropriate development platform based on your needs;

• Develop the product according to its functional requirements. If any issues arise during the development process, please contact Jidong Intelligent staff promptly;

• Finalize the mass production plan for the final product.

Usage Recommendations

Factors affecting ranging capability and accuracy include:

- Target reflectivity: Generally, the higher the target reflectivity, the better the ranging capability and the faster the response speed. For example, targets with medium reflectivity can be measured up to 1500 m, while targets with low reflectivity may only be measured up to 800 m.

- Measurement angle: When the laser angle is perpendicular to the target, the ranging capability is better and the response speed is faster; otherwise, it decreases. Under extreme conditions, it cannot be guaranteed that the ranging capability and response speed specified in this manual can be achieved.

- Measurement environment: Factors affecting ranging capability and response speed also include sunlight intensity, the concentration of water vapor and suspended particles in the air, and the angle deviating from direct sunlight.